Caution:  The appropriate update file must be used for this update process and depends on the current version of your VG-99.  Attempting to update the VG-99 with the incorrect files could cause possible damage.

Use the following steps to check the current version number of your VG-99:

While holding the CATEGORY and PAGE Left (< ) buttons, turn on the power. The current version will show in the display next to “Firm:”.

If the current version is 1.03, use files in folder “vg99_v104_for103” and execute it.

If the current version is 1.01  or 1.02, use files in folder “vg99_v104_for102” and  execute it.

Never turn off the power while the update is in process.

System Update Instructions

This procedure updates the VG-99’s system using a computer or other such

device to play back the MIDI data for update, which is then received by the

VG-99.

Required Items

• PC (Windows XP/2000)

• Software Sequencer: UpdSMF.exe

(Refer to Service Information #102333.)

System Update Procedure

1. Connect the computer’s MIDI OUT to the VG-99’s MIDI IN (use a UM-1 USB MIDI interface or equivalent device).

2. Simultaneously hold down PAGE left < and CATEGORY and turn on the

power.

“SYSTEM UPDATER Please wait...” appears in the display.

3. Wait several seconds until “Waiting SMF...” is displayed.

4. Start up the application for transmitting MIDI data on the computer (e.g.,

UpdSMF), and transmit the MIDI data.

“Now Receiving...” is displayed while receiving data.

* The update takes approximately 30 minutes.

When the update is finished, “Complete!” appears in the display.

Caution: Never turn off the power while the update is in process.